PM-1000/CRV1000 SPECIFICATIONS

Power supply:	A.C. 117V +15% / -25%, 220V optional

Compatible Probes:	pH & ORP High-Z voltage output.

Probe Selection:	Internal switch (pH/ORP select)

pH Display Range:	0 to 14

pH Display Resolution:	0.01pH

pH Voltage Input:	+/- 550mV

pH Accuracy:	+/- 0.01pH

ORP Display Range:	-2000mV to 2000mV

ORP Display Resolution:	1mV

ORP Voltage Input:	+/-2200mV

ORP Accuracy:	1mV

Temperature Input Range:	0 to 100˚C, 32 to 212˚F

Temperature Display Resolution:	1 ˚C/˚F

Temperature Sensor:	1000 Ω Pt RTD

Temperature Accuracy:	+/-1˚C/˚F

Analog Output Type:	4-20mA

Analog Output Compliance:	600 Ω

Resolution:	12 bit

Isolation:	Galvanic/Optical

Breakdown Voltage:	1500VAC.

Relay Outputs:	Two, Form-A

Functions:	Low & High set points

Voltage:	250V A.C./D.C.

D.C. & A.C. Current:	5A (resistive)

Display:	Five, 7-segment, red, 0.56”

LED bars:	Up to 8

Pushbuttons:	Three

ROM Memory:	32k, FLASH

Field Upgradeable:	Yes (via RS232 port)

Serial Communications:	RS232*

Operating Temperature Range:	-10˚C to 50˚C / -14˚F to 122˚F

Humidity:	10%, non-condensing

*Requires isolation module if used permanently.